I. DESCRIPTION

This specification applies for motor KSV 5035/757, part No. 94 11 81 07. The motor has been designed

mechanically and electrically for electrode feed in welding machines and is equipped with OPTICAL encoder.

With its good properties and high reliability the motor can also be used in many other applications.

II. MECHANICAL DATA

1. Housing ( Gear box ):  The motor has a housing of aluminium with high strength and good corrosion

resistance. The housing is provided with 3 studs for M6 fixing screws.

2. Shafts and gear wheel: . The gear wheel shaft has good anti-friction properties. The casehardening of the gear wheel shaft guarantees long useful life and effective torque transmission. The gear is made of plastic , and has gear quality 10 fs acc. to DIN 3967.

3. Bearing and lubrication: The shaft of the armature is mounted with a single row deep grove ball

bearing type 2Z and one plain bearing. The gear wheel is mounted with 2 needle bearings.

| III. TECHNICAL DATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Technical data | Design. | Unit | Value |
| 1 | Rated voltage | Un | V DC | 24 |
| 2 | Gear ratio ( 2 tooth worm ) | i | n1/n2 | 24 |
| 3 | Radial load on the shaft | Frad | N | 600 max |
| 4 | Test data at no-load - after 20 min. unloaded run in ( Ref. only ): |  |  |  |
| 4.1 | Speed | n | rpm | 157±10% |
| 4.2 | Current | I | A | 1,0 max |
| 5 | FHP Test data - after 20 min. unloaded run in: |  |  |  |
| 5.1 | Torque | Mt | Nm | 1.5 |
| 5.2 | Speed | nt | rpm | 150±10% |
| 5.3 | Current | It | A | 2.8 max |
| 6 | Continuous operation |  |  |  |
|  | Data for expected lifetime , maximum allowed torque , running conditions , etc. to be evaluated depending on application. |  |  |  |
| 7 | Intermittent operation |  |  |  |
|  | Data for expected lifetime , maximum allowed torque , running conditions , etc. to be evaluated depending on application. |  |  |  |
| 8 | Degree of protection | - | - | IP20 |
| 9 | Motor weight | - | kg | 1.71 |
| 10 | Optical encoder | - | - | yes |
| 10.1 | Pulses | - | Number / turn | 60 |
| 10.2 | Output | - | - | 1 |
| 10.3 | Protection | - | - | No |
| 10.4 | Line driver | - | - | No |