________________________________________________________________________________

## Produktinformation/

Product Information

Eichendorffstraße 28

D-78054 VS-Schwenningen

Telefon	+49 (0) 7720 3002-0

Telefax	+49 (0) 7720 3002-11

sales@barigo.de

| # Artikel/Article | ### Mechanisches Thermo- Hygrometer |  |  |
| --- | --- | --- | --- |
| Artikel -Nr./Article No. | 2300 |  |  |
| Netto EK/Purchase Price net |  |  |  |
| Beschreibung/Description | Thermometer & Hygrometer |  |  |
| Technische Daten: |  | Technical Specifications: |  |
| Thermometer - Skala von -20°C…+50°C - Auflösung 1°C Hygrometer - Skala von 10% RH…100% RH - Auflösung 1% RH Genauigkeit - Thermometer: +- 1°C im Bereich 0°C…40°C - Hygrometer: +/- 5% im Bereich 40% …70% RH Gehäuse - Material: Messing verchromt - Maße: 130 x 35mm - Mineralglas - Gewicht: 210g EAN Code: NN Zolltarif-Nr.: 90258080 MADE IN GERMANY |  | Thermometer - Measuring range -20°C…+50°C - Resolution 1°C Hygrometer - Measuring range 10% RH…100%RH - Resolution 1%RH Accuracy - Thermometer: +- 1°C in the range 0°C…40°C - Hygrometer: +/- 5% in the range of 40% …70% RH Case - Material: chromed brass - Measures: 130 x 35mm - Mineral glass - Weight: 210g EAN Code: NN Custom tariff code: 90258080 MADE IN GERMANY |  |